# DHCP Pool Configurations

DNS Strategy: Internal DNS server (10.10.10.11) as primary, Google DNS (8.8.8.8) as fallback.

Lease Time: 4 hours for all client VLANs.  |  Exclusions: .1–.5 reserved in every subnet.

VLAN 10 Management: Fully static — no DHCP pool. All management devices use hardcoded IPs.

## 1. DHCP Pool Summary

| VLAN | Name | Subnet | Excluded (.1–.5) | Dynamic Range | Served By | Lease |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Management | 10.10.10.0/24 | 10.10.10.1–.5 | NO POOL — all static | N/A | — |
| 20 | Staff | 10.10.20.0/24 | 10.10.20.1–.5 | 10.10.20.6–.254 | SW-CORE* | 4 hrs |
| 30 | Labs (GF) | 10.10.30.0/24 | 10.10.30.1–.5 | 10.10.30.6–.254 | SW-GF-DIST | 4 hrs |
| 40 | Wireless | 10.10.40.0/24 | 10.10.40.1–.5 | 10.10.40.6–.254 | Per floor DIST | 4 hrs |
| 50 | Security | 10.10.50.0/24 | 10.10.50.1–.5 | 10.10.50.6–.254 | SW-CORE | 4 hrs |
| 60 | Access-Control | 10.10.60.0/24 | 10.10.60.1–.5 | 10.10.60.6–.254 | Per floor DIST | 4 hrs |
| 70 | SoftwareLab | 10.10.70.0/24 | 10.10.70.1–.5 | 10.10.70.6–.254 | SW-1F-DIST | 4 hrs |
| 80 | ComputerLab | 10.10.80.0/24 | 10.10.80.1–.5 | 10.10.80.6–.254 | SW-1F-DIST | 4 hrs |
| 99 | Guest | 10.10.99.0/24 | 10.10.99.1–.5 | 10.10.99.6–.254 | Per floor DIST | 4 hrs |

* VLAN 20 Staff spans both floors so SW-CORE owns the pool. Both 1F-DIST and GF-DIST use ip helper-address to relay requests up to SW-CORE.

## 2. Static IP Reservations (VLAN 10)

| Device | VLAN | Static IP | Notes |
| --- | --- | --- | --- |
| SW-CORE | 10 | 10.10.10.1 | Core L3 — also default gateway for all VLANs |
| SW-1F-DIST | 10 | 10.10.10.2 | 1F distribution — default GW for 1F access switches |
| SW-GF-DIST | 10 | 10.10.10.3 | GF distribution — default GW for GF access switches |
| SW-SoftLab | 10 | 10.10.10.4 | 1F Software Lab access switch |
| SW-CompLab | 10 | 10.10.10.5 | 1F Computer Lab access switch |
| SW-Office-Cam | 10 | 10.10.10.6 | 1F Offices access switch |
| SW-GF-LabCL1 | 10 | 10.10.10.7 | GF Lab + CL1 access switch |
| SW-GF-Office | 10 | 10.10.10.8 | GF Offices access switch |
| DHCP Server | 10 | 10.10.10.10 | Server-PT — not used for DHCP (switches handle it) |
| DNS Server | 10 | 10.10.10.11 | Internal DNS — primary for all client pools |
| RADIUS/NVR Server | 10 | 10.10.10.12 | RADIUS for 802.1X + NVR for cameras |
| ISA 3000 Firewall (inside) | 10 | 10.10.10.254 | Inside interface — also core uplink gateway |

## 3. SW-CORE DHCP Configuration

SW-CORE owns: VLAN 20 Staff, VLAN 50 Security. It also receives relayed requests from distribution switches for these pools.

! ─── EXCLUSIONS ───────────────────────────────────────────

! Exclude static device range (.1–.5) from every subnet

ip dhcp excluded-address 10.10.10.1 10.10.10.5

ip dhcp excluded-address 10.10.20.1 10.10.20.5

ip dhcp excluded-address 10.10.50.1 10.10.50.5

! ─── POOL: VLAN 20 — Staff ────────────────────────────────

ip dhcp pool VLAN20-Staff

network 10.10.20.0 255.255.255.0

default-router 10.10.20.1

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name staff.local

! ─── POOL: VLAN 50 — Security (Cameras/NVR) ───────────────

! Cameras use DHCP for simplicity; NVR is static at .12

ip dhcp pool VLAN50-Security

network 10.10.50.0 255.255.255.0

default-router 10.10.50.1

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name security.local

## 4. SW-1F-DIST DHCP Configuration

SW-1F-DIST owns: VLAN 40 Wireless (1F APs), VLAN 60 Access-Control (1F RFID), VLAN 70 SoftwareLab, VLAN 80 ComputerLab, VLAN 99 Guest (1F).

It also relays VLAN 20 Staff requests up to SW-CORE using ip helper-address.

! ─── EXCLUSIONS ───────────────────────────────────────────

ip dhcp excluded-address 10.10.40.1 10.10.40.5

ip dhcp excluded-address 10.10.60.1 10.10.60.5

ip dhcp excluded-address 10.10.70.1 10.10.70.5

ip dhcp excluded-address 10.10.80.1 10.10.80.5

ip dhcp excluded-address 10.10.99.1 10.10.99.5

! ─── POOL: VLAN 40 — Wireless (1F APs) ───────────────────

ip dhcp pool VLAN40-Wireless-1F

network 10.10.40.0 255.255.255.0

default-router 10.10.40.2

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name wireless.local

! ─── POOL: VLAN 60 — Access Control (1F RFID) ─────────────

ip dhcp pool VLAN60-AccessControl-1F

network 10.10.60.0 255.255.255.0

default-router 10.10.60.2

dns-server 10.10.10.11

lease 0 4

domain-name access.local

! ─── POOL: VLAN 70 — Software Lab ─────────────────────────

ip dhcp pool VLAN70-SoftwareLab

network 10.10.70.0 255.255.255.0

default-router 10.10.70.2

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name softlab.local

! ─── POOL: VLAN 80 — Computer Lab ─────────────────────────

ip dhcp pool VLAN80-ComputerLab

network 10.10.80.0 255.255.255.0

default-router 10.10.80.2

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name complab.local

! ─── POOL: VLAN 99 — Guest (1F) ───────────────────────────

! Guest DNS points to public only — no internal DNS exposure

ip dhcp pool VLAN99-Guest-1F

network 10.10.99.0 255.255.255.0

default-router 10.10.99.2

dns-server 8.8.8.8 8.8.4.4

lease 0 4

! ─── HELPER ADDRESS for VLAN 20 Staff ─────────────────────

! Relay Staff DHCP requests to SW-CORE SVI 10.10.20.1

interface vlan 20

ip helper-address 10.10.20.1

## 5. SW-GF-DIST DHCP Configuration

SW-GF-DIST owns: VLAN 30 Labs (GF), VLAN 40 Wireless (GF APs), VLAN 60 Access-Control (GF RFID), VLAN 99 Guest (GF).

It also relays VLAN 20 Staff requests up to SW-CORE using ip helper-address.

! ─── EXCLUSIONS ───────────────────────────────────────────

ip dhcp excluded-address 10.10.30.1 10.10.30.5

ip dhcp excluded-address 10.10.40.1 10.10.40.5

ip dhcp excluded-address 10.10.60.1 10.10.60.5

ip dhcp excluded-address 10.10.99.1 10.10.99.5

! ─── POOL: VLAN 30 — Labs (GF Comp Eng + CL1) ─────────────

ip dhcp pool VLAN30-Labs-GF

network 10.10.30.0 255.255.255.0

default-router 10.10.30.3

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name labs.local

! ─── POOL: VLAN 40 — Wireless (GF APs) ───────────────────

ip dhcp pool VLAN40-Wireless-GF

network 10.10.40.0 255.255.255.0

default-router 10.10.40.3

dns-server 10.10.10.11 8.8.8.8

lease 0 4

domain-name wireless.local

! ─── POOL: VLAN 60 — Access Control (GF RFID) ─────────────

ip dhcp pool VLAN60-AccessControl-GF

network 10.10.60.0 255.255.255.0

default-router 10.10.60.3

dns-server 10.10.10.11

lease 0 4

domain-name access.local

! ─── POOL: VLAN 99 — Guest (GF) ───────────────────────────

! Guest DNS points to public only — no internal DNS exposure

ip dhcp pool VLAN99-Guest-GF

network 10.10.99.0 255.255.255.0

default-router 10.10.99.3

dns-server 8.8.8.8 8.8.4.4

lease 0 4

! ─── HELPER ADDRESS for VLAN 20 Staff ─────────────────────

! Relay Staff DHCP requests to SW-CORE SVI 10.10.20.1

interface vlan 20

ip helper-address 10.10.20.1

## 6. Important Note — VLAN 40 Wireless

VLAN 40 exists on both floors but uses the same subnet (10.10.40.0/24). Each distribution switch has its own SVI (.2 for 1F, .3 for GF) and serves as the default gateway for APs on its floor. Both pools draw from the same /24 so ensure AP counts across both floors stay well under 249 combined devices.

If you want to split them into separate subnets in the future:

! Option: 1F APs  → 10.10.40.0/25  (.6–.126)

! Option: GF APs  → 10.10.40.128/25 (.134–.254)

## 7. Guest VLAN 99 — Isolation Reminder

Guest DHCP pools intentionally use only public DNS (8.8.8.8 / 8.8.4.4) — internal DNS server is never handed to guest devices. You must also configure an ACL on SW-CORE to block VLAN 99 from reaching any internal subnet:

! On SW-CORE — block guest from all internal VLANs

ip access-list extended BLOCK-GUEST-INTERNAL

deny ip 10.10.99.0 0.0.0.255 10.10.0.0 0.0.255.255

permit ip any any

interface vlan 99

ip access-group BLOCK-GUEST-INTERNAL in

## 8. Verification Commands

Use these on any L3 switch to confirm DHCP is working correctly:

! View all active DHCP leases

show ip dhcp binding

! Check pool utilisation and conflicts

show ip dhcp pool

! View excluded address ranges

show ip dhcp excluded-address

! Check for DHCP conflicts (should be empty)

show ip dhcp conflict

! Confirm helper-address is set on VLAN 20 SVIs

show running-config | section interface vlan 20

! Debug DHCP in real time (use carefully — verbose output)

debug ip dhcp server events