# RADIUS & TACACS+ AAA Configuration

RADIUS handles end-user and RFID port authentication (802.1X + MAB fallback).

TACACS+ handles switch administrator login and per-command authorization over SSH.

Both services run on the RADIUS/NVR server at 10.10.10.12 (Server-PT in Packet Tracer).

## 1. Protocol Overview

| Protocol | Purpose | Server IP | Port | Used By |
| --- | --- | --- | --- | --- |
| RADIUS | End-user + RFID 802.1X/MAB authentication | 10.10.10.12 | 1812/1813 | All access switches |
| TACACS+ | Switch admin login + command authorization | 10.10.10.12 | 49 | All L3 + access switches |

## 2. Access Tier Policy

| Tier | VLAN(s) | RADIUS Policy | Can Reach |
| --- | --- | --- | --- |
| Staff | VLAN 20 | ACL: STAFF-ACCESS | All internal VLANs + internet |
| Lab User | VLAN 30 / 70 / 80 | ACL: LAB-ACCESS | Own lab VLAN + internet only |
| RFID/Access | VLAN 60 | ACL: RFID-ACCESS | VLAN 60 + RADIUS server only |
| Guest | VLAN 99 | No RADIUS — open DHCP | Internet only (blocked from internal) |
| Failed Auth | VLAN 60 (stays open) | Alert logged, port stays up | Unchanged — no VLAN change |

## 3. RADIUS/NVR Server-PT Setup (Packet Tracer)

In Packet Tracer, configure the RADIUS and TACACS+ services on the Server-PT device (10.10.10.12):

RADIUS Service (AAA tab):

Network Configuration (on Server-PT AAA tab):

Client Name : SW-CORE         | Client IP: 10.10.10.1  | Secret: CiscoRAD123!

Client Name : SW-1F-DIST      | Client IP: 10.10.10.2  | Secret: CiscoRAD123!

Client Name : SW-GF-DIST      | Client IP: 10.10.10.3  | Secret: CiscoRAD123!

Client Name : SW-SoftLab      | Client IP: 10.10.10.4  | Secret: CiscoRAD123!

Client Name : SW-CompLab      | Client IP: 10.10.10.5  | Secret: CiscoRAD123!

Client Name : SW-Office-Cam   | Client IP: 10.10.10.6  | Secret: CiscoRAD123!

Client Name : SW-GF-LabCL1    | Client IP: 10.10.10.7  | Secret: CiscoRAD123!

Client Name : SW-GF-Office    | Client IP: 10.10.10.8  | Secret: CiscoRAD123!

User Accounts (on Server-PT AAA tab — User Setup):

Username: staff-user     | Password: Staff@2025  | (represents VLAN 20 staff)

Username: lab-user       | Password: Lab@2025    | (represents VLAN 30/70/80 lab)

Username: rfid-reader    | Password: RFID@2025   | (represents VLAN 60 RFID devices)

TACACS+ Service (AAA tab):

TACACS+ Configuration (on Server-PT AAA tab):

Client IP : 10.10.10.1 – 10.10.10.8 (all switches)  | Secret: CiscoTAC123!

Admin Accounts:

Username: netadmin   | Password: Admin@2025  | Privilege: 15 (full access)

Username: netmon     | Password: Mon@2025    | Privilege: 5  (show commands only)

## 4. SW-CORE — AAA Base Configuration

SW-CORE defines the global AAA policy. All switches reference the same RADIUS and TACACS+ server. Configure this first — other switches follow the same AAA base.

! ─── ENABLE AAA ───────────────────────────────────────────

aaa new-model

! ─── RADIUS SERVER ────────────────────────────────────────

radius server RADIUS-SRV

address ipv4 10.10.10.12 auth-port 1812 acct-port 1813

key CiscoRAD123!

! ─── TACACS+ SERVER ───────────────────────────────────────

tacacs server TACACS-SRV

address ipv4 10.10.10.12

key CiscoTAC123!

! ─── SERVER GROUPS ────────────────────────────────────────

aaa group server radius RADIUS-GROUP

server name RADIUS-SRV

aaa group server tacacs+ TACACS-GROUP

server name TACACS-SRV

! ─── AUTHENTICATION POLICIES ──────────────────────────────

! 802.1X port auth — use RADIUS, fall back to local

aaa authentication dot1x default group RADIUS-GROUP local

! Device admin login — TACACS+ primary, local fallback

aaa authentication login ADMIN-AUTH group TACACS-GROUP local

! Enable mode — TACACS+ primary, local fallback

aaa authentication enable default group TACACS-GROUP enable

! ─── AUTHORIZATION POLICIES ───────────────────────────────

! Network 802.1X/MAB — RADIUS assigns VLAN/ACL dynamically

aaa authorization network default group RADIUS-GROUP

! Exec (SSH shell) — TACACS+ authorizes privilege level

aaa authorization exec ADMIN-AUTHOR group TACACS-GROUP local

! Per-command authorization — TACACS+ checks every command

aaa authorization commands 15 default group TACACS-GROUP local

! ─── ACCOUNTING ───────────────────────────────────────────

! Log all 802.1X auth events to RADIUS

aaa accounting dot1x default start-stop group RADIUS-GROUP

! Log all admin commands to TACACS+

aaa accounting exec default start-stop group TACACS-GROUP

aaa accounting commands 15 default start-stop group TACACS-GROUP

! ─── GLOBAL 802.1X ────────────────────────────────────────

dot1x system-auth-control

! ─── LOCAL FALLBACK ADMIN ACCOUNT ─────────────────────────

! Used if TACACS+ server is unreachable

username netadmin privilege 15 secret Admin@2025

! ─── VTY LINES — SSH only, TACACS+ auth ───────────────────

line vty 0 4

login authentication ADMIN-AUTH

authorization exec ADMIN-AUTHOR

transport input ssh

## 5. Access Switch — 802.1X + MAB Port Configuration

Apply this to every RFID reader port (VLAN 60) on all access switches. The switch tries 802.1X first; if the device does not respond within the timeout, it falls back to MAB (authenticates by MAC address). On failure, the port stays open and logs an alert — no VLAN change.

### 5a. AAA Base (copy to every access switch — same as SW-CORE)

aaa new-model

radius server RADIUS-SRV

address ipv4 10.10.10.12 auth-port 1812 acct-port 1813

key CiscoRAD123!

tacacs server TACACS-SRV

address ipv4 10.10.10.12

key CiscoTAC123!

aaa group server radius RADIUS-GROUP

server name RADIUS-SRV

aaa group server tacacs+ TACACS-GROUP

server name TACACS-SRV

aaa authentication dot1x default group RADIUS-GROUP local

aaa authentication login ADMIN-AUTH group TACACS-GROUP local

aaa authorization network default group RADIUS-GROUP

aaa authorization exec ADMIN-AUTHOR group TACACS-GROUP local

aaa accounting dot1x default start-stop group RADIUS-GROUP

dot1x system-auth-control

username netadmin privilege 15 secret Admin@2025

line vty 0 4

login authentication ADMIN-AUTH

authorization exec ADMIN-AUTHOR

transport input ssh

### 5b. RFID Port Config (VLAN 60 ports on every access switch)

! ── SW-SoftLab example — FastEthernet0/10 (RFID reader) ───

! Repeat this block for every RFID port across all switches

interface FastEthernet0/10

description RFID-Reader-VLAN60

switchport mode access

switchport access vlan 60

spanning-tree portfast

power inline auto

! Enable 802.1X on this port

authentication port-control auto

authentication order dot1x mab

authentication priority dot1x mab

! Try 802.1X first; wait 10s before falling back to MAB

dot1x pae authenticator

dot1x timeout tx-period 10

mab

! On auth failure: keep port open, log alert only

authentication event fail action next-method

authentication event no-response action authorize vlan 60

! Re-authenticate every 3600 seconds (1 hour)

authentication timer reauthenticate 3600

no shutdown

## 6. Dynamic ACL Policy (RADIUS Returns Per-User ACL)

When RADIUS authenticates a user/RFID card, it returns a Cisco-AV-Pair attribute that assigns a named ACL to that session. Define these ACLs on SW-CORE and all distribution switches.

### 6a. Staff Access ACL — full internal access

ip access-list extended STAFF-ACCESS

permit ip 10.10.20.0 0.0.0.255 any

permit ip any any

### 6b. Lab User ACL — own lab subnet + internet only

ip access-list extended LAB-ACCESS

! Allow lab subnets to reach internet (via firewall)

permit ip 10.10.30.0 0.0.0.255 any

permit ip 10.10.70.0 0.0.0.255 any

permit ip 10.10.80.0 0.0.0.255 any

! Block lab from reaching staff, security or mgmt VLANs

deny ip any 10.10.20.0 0.0.0.255

deny ip any 10.10.50.0 0.0.0.255

deny ip any 10.10.60.0 0.0.0.255

deny ip any 10.10.10.0 0.0.0.255

permit ip any any

### 6c. RFID/Access-Control ACL — access control plane only

ip access-list extended RFID-ACCESS

! RFID readers only need to reach the RADIUS/NVR server

permit ip 10.10.60.0 0.0.0.255 host 10.10.10.12

! Allow intra-VLAN 60 communication (reader-to-controller)

permit ip 10.10.60.0 0.0.0.255 10.10.60.0 0.0.0.255

! Block everything else

deny ip any any

### 6d. RADIUS AV-Pair attributes (configure on Server-PT per user)

! In Server-PT AAA tab, set these Cisco-AV-Pair values per user:

staff-user  → Cisco-AV-Pair = shell:priv-lvl=1

Cisco-AV-Pair = ip:inacl=STAFF-ACCESS

lab-user    → Cisco-AV-Pair = ip:inacl=LAB-ACCESS

Tunnel-Type = VLAN

Tunnel-Medium-Type = 802

Tunnel-Private-Group-ID = 30   (or 70 or 80)

rfid-reader → Cisco-AV-Pair = ip:inacl=RFID-ACCESS

Tunnel-Type = VLAN

Tunnel-Medium-Type = 802

Tunnel-Private-Group-ID = 60

## 7. TACACS+ Command Authorization Levels

TACACS+ enforces what each admin account can do once logged in. Privilege 15 = full access. Privilege 5 = monitoring only (show commands).

! ─── PRIVILEGE LEVELS ─────────────────────────────────────

privilege exec level 5 show

privilege exec level 5 ping

privilege exec level 5 traceroute

! ─── TACACS+ AUTHORIZATION FOR PRIV 15 ───────────────────

aaa authorization commands 15 default group TACACS-GROUP local

! ─── TACACS+ AUTHORIZATION FOR PRIV 5 ────────────────────

aaa authorization commands 5 default group TACACS-GROUP local

! On Server-PT, set privilege level per admin:

!   netadmin → privilege 15 (full)

!   netmon   → privilege 5  (show only)

## 8. Verification Commands

### RADIUS / 802.1X

! View all authenticated 802.1X/MAB sessions

show authentication sessions

! Detail for a specific interface

show authentication sessions interface FastEthernet0/10

! View 802.1X statistics

show dot1x all statistics

! Check RADIUS server reachability

show radius server-group all

! Debug 802.1X auth in real time (use carefully)

debug dot1x all

debug radius authentication

### TACACS+

! Confirm TACACS+ server is reachable

show tacacs

! View AAA method lists

show aaa method-lists

! View active admin sessions

show users

! Check accounting log (if local logging enabled)

show aaa accounting

! Debug TACACS+ login

debug tacacs

debug aaa authentication

debug aaa authorization

## 9. Auth Flow Summary

| Step | Event | Action |
| --- | --- | --- |
| 1 | RFID reader port comes up | Switch sends EAP-Request/Identity (802.1X) |
| 2 | Device responds with 802.1X | Switch relays to RADIUS — authenticates by credentials |
| 3 | Device silent (no 802.1X supplicant) | Switch falls back to MAB — sends MAC to RADIUS |
| 4 | RADIUS returns Access-Accept | Switch assigns VLAN 60 + RFID-ACCESS ACL to session |
| 5 | RADIUS returns Access-Reject | Port stays open on VLAN 60, syslog alert generated |
| 6 | Admin SSHs into switch | TACACS+ authenticates login + authorizes each command |
| 7 | Session re-auth timer fires (1hr) | Switch silently re-authenticates active sessions |