# Network Switch Configurations

All L3 switches: Cisco 3650-24PS   |   All access switches: Cisco 2960

## VLAN Reference Table

| VLAN | Name | Subnet | Used By |
| --- | --- | --- | --- |
| 10 | Management | 10.10.10.0/24 | All switch management IPs |
| 20 | Staff | 10.10.20.0/24 | Office PCs |
| 30 | Labs | 10.10.30.0/24 | Comp Eng Lab + CL1 PCs (GF) |
| 40 | Wireless | 10.10.40.0/24 | All APs (management traffic) |
| 50 | Security | 10.10.50.0/24 | IP cameras, NVR |
| 60 | Access-Control | 10.10.60.0/24 | RFID readers |
| 70 | SoftwareLab | 10.10.70.0/24 | Software Lab PCs |
| 80 | ComputerLab | 10.10.80.0/24 | Computer Lab PCs |
| 99 | Guest | 10.10.99.0/24 | Guest Wi-Fi (isolated) |

## Management IP Summary

| Switch | Management IP | Default Gateway |
| --- | --- | --- |
| SW-CORE | 10.10.10.1 | — (is the core) |
| SW-1F-DIST | 10.10.10.2 | 10.10.10.1 |
| SW-GF-DIST | 10.10.10.3 | 10.10.10.1 |
| SW-SoftLab | 10.10.10.4 | 10.10.10.2 |
| SW-CompLab | 10.10.10.5 | 10.10.10.2 |
| SW-Office-Cam | 10.10.10.6 | 10.10.10.2 |
| SW-GF-LabCL1 | 10.10.10.7 | 10.10.10.3 |
| SW-GF-Office | 10.10.10.8 | 10.10.10.3 |

## SW-CORE — Cisco 3650-24PS (Server Room)

Role: Core routing, firewall uplink, all servers, SMF uplinks to both distribution switches.

ip routing

vlan 10

name Management

vlan 20

name Staff

vlan 30

name Labs

vlan 40

name Wireless

vlan 50

name Security

vlan 60

name Access-Control

vlan 70

name SoftwareLab

vlan 80

name ComputerLab

vlan 99

name Guest

! SVIs — core owns the gateway for all VLANs

interface vlan 10

ip address 10.10.10.1 255.255.255.0

no shutdown

interface vlan 20

ip address 10.10.20.1 255.255.255.0

no shutdown

interface vlan 30

ip address 10.10.30.1 255.255.255.0

no shutdown

interface vlan 40

ip address 10.10.40.1 255.255.255.0

no shutdown

interface vlan 50

ip address 10.10.50.1 255.255.255.0

no shutdown

interface vlan 60

ip address 10.10.60.1 255.255.255.0

no shutdown

interface vlan 70

ip address 10.10.70.1 255.255.255.0

no shutdown

interface vlan 80

ip address 10.10.80.1 255.255.255.0

no shutdown

interface vlan 99

ip address 10.10.99.1 255.255.255.0

no shutdown

! Uplink to ISA 3000 firewall

interface GigabitEthernet1/0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,60,70,80,99

no shutdown

! SMF uplink to SW-1F-DIST (SFP port)

interface GigabitEthernet1/1/1

switchport mode trunk

switchport trunk allowed vlan 10,20,40,50,60,70,80,99

no shutdown

! SMF uplink to SW-GF-DIST (SFP port)

interface GigabitEthernet1/1/2

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,60,99

no shutdown

! Server ports — VLAN 10 (DHCP, DNS, RADIUS, NVR)

interface range GigabitEthernet1/0/2 - 5

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

! Shut unused

interface range GigabitEthernet1/0/6 - 24

shutdown

line vty 0 4

transport input ssh

login local

## SW-1F-DIST — Cisco 3650-24PS (First Floor Distribution)

Role: Full L3 inter-VLAN routing for all first floor VLANs. SMF uplink to core. Default route to core for WAN/inter-floor traffic.

ip routing

vlan 10

name Management

vlan 20

name Staff

vlan 40

name Wireless

vlan 50

name Security

vlan 60

name Access-Control

vlan 70

name SoftwareLab

vlan 80

name ComputerLab

vlan 99

name Guest

! SVIs — routes 1F VLANs locally

interface vlan 10

ip address 10.10.10.2 255.255.255.0

no shutdown

interface vlan 20

ip address 10.10.20.2 255.255.255.0

no shutdown

interface vlan 40

ip address 10.10.40.2 255.255.255.0

no shutdown

interface vlan 50

ip address 10.10.50.2 255.255.255.0

no shutdown

interface vlan 60

ip address 10.10.60.2 255.255.255.0

no shutdown

interface vlan 70

ip address 10.10.70.2 255.255.255.0

no shutdown

interface vlan 80

ip address 10.10.80.2 255.255.255.0

no shutdown

interface vlan 99

ip address 10.10.99.2 255.255.255.0

no shutdown

! SMF uplink to SW-CORE (SFP port)

interface GigabitEthernet1/1/1

switchport mode trunk

switchport trunk allowed vlan 10,20,40,50,60,70,80,99

no shutdown

! Downlink to SW-SoftLab

interface GigabitEthernet1/0/1

switchport mode trunk

switchport trunk allowed vlan 10,40,50,60,70,99

no shutdown

! Downlink to SW-CompLab

interface GigabitEthernet1/0/2

switchport mode trunk

switchport trunk allowed vlan 10,40,50,60,80,99

no shutdown

! Downlink to SW-Office-Cam

interface GigabitEthernet1/0/3

switchport mode trunk

switchport trunk allowed vlan 10,20,50,99

no shutdown

! Default route to core for WAN and inter-floor traffic

ip route 0.0.0.0 0.0.0.0 10.10.10.1

! Shut unused

interface range GigabitEthernet1/0/4 - 24

shutdown

line vty 0 4

transport input ssh

login local

## SW-GF-DIST — Cisco 3650-24PS (Ground Floor Distribution)

Role: Full L3 inter-VLAN routing for all ground floor VLANs. SMF uplink to core. Both GF access switches connect directly here — no daisy chaining.

ip routing

vlan 10

name Management

vlan 20

name Staff

vlan 30

name Labs

vlan 40

name Wireless

vlan 50

name Security

vlan 60

name Access-Control

vlan 99

name Guest

! SVIs — routes GF VLANs locally

interface vlan 10

ip address 10.10.10.3 255.255.255.0

no shutdown

interface vlan 20

ip address 10.10.20.3 255.255.255.0

no shutdown

interface vlan 30

ip address 10.10.30.3 255.255.255.0

no shutdown

interface vlan 40

ip address 10.10.40.3 255.255.255.0

no shutdown

interface vlan 50

ip address 10.10.50.3 255.255.255.0

no shutdown

interface vlan 60

ip address 10.10.60.3 255.255.255.0

no shutdown

interface vlan 99

ip address 10.10.99.3 255.255.255.0

no shutdown

! SMF uplink to SW-CORE (SFP port)

interface GigabitEthernet1/1/1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,40,50,60,99

no shutdown

! Independent downlink to SW-GF-LabCL1

interface GigabitEthernet1/0/1

switchport mode trunk

switchport trunk allowed vlan 10,30,40,50,60,99

no shutdown

! Independent downlink to SW-GF-Office

interface GigabitEthernet1/0/2

switchport mode trunk

switchport trunk allowed vlan 10,20,50,99

no shutdown

! Default route to core for WAN and inter-floor traffic

ip route 0.0.0.0 0.0.0.0 10.10.10.1

! Shut unused

interface range GigabitEthernet1/0/3 - 24

shutdown

line vty 0 4

transport input ssh

login local

## SW-SoftLab — Cisco 2960 (1F Software Lab)

Uplink to SW-1F-DIST. Carries VLANs: 10, 40, 50, 60, 70, 99. Default gateway: 10.10.10.2

vlan 10

name Management

vlan 40

name Wireless

vlan 50

name Security

vlan 60

name Access-Control

vlan 70

name SoftwareLab

vlan 99

name Guest

! Uplink to SW-1F-DIST

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,40,50,60,70,99

no shutdown

! AP port — trunk: VLAN 40 (AP mgmt) + VLAN 70 (lab) + VLAN 99 (guest)

interface FastEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 40,70,99

spanning-tree portfast

! Software Lab PC ports — VLAN 70

interface range FastEthernet0/2 - 8

switchport mode access

switchport access vlan 70

spanning-tree portfast

! Camera port — PoE, VLAN 50

interface FastEthernet0/9

switchport mode access

switchport access vlan 50

spanning-tree portfast

power inline auto

! RFID reader port — PoE, VLAN 60

interface FastEthernet0/10

switchport mode access

switchport access vlan 60

spanning-tree portfast

power inline auto

! Shut unused

interface range FastEthernet0/11 - 24

shutdown

interface vlan 10

ip address 10.10.10.4 255.255.255.0

no shutdown

ip default-gateway 10.10.10.2

line vty 0 4

transport input ssh

login local

## SW-CompLab — Cisco 2960 (1F Computer Lab)

Uplink to SW-1F-DIST. Carries VLANs: 10, 40, 50, 60, 80, 99. Default gateway: 10.10.10.2

vlan 10

name Management

vlan 40

name Wireless

vlan 50

name Security

vlan 60

name Access-Control

vlan 80

name ComputerLab

vlan 99

name Guest

! Uplink to SW-1F-DIST

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,40,50,60,80,99

no shutdown

! AP port — trunk: VLAN 40 (AP mgmt) + VLAN 80 (lab) + VLAN 99 (guest)

interface FastEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 40,80,99

spanning-tree portfast

! Computer Lab PC ports — VLAN 80

interface range FastEthernet0/2 - 8

switchport mode access

switchport access vlan 80

spanning-tree portfast

! Camera port — PoE, VLAN 50

interface FastEthernet0/9

switchport mode access

switchport access vlan 50

spanning-tree portfast

power inline auto

! RFID reader port — PoE, VLAN 60

interface FastEthernet0/10

switchport mode access

switchport access vlan 60

spanning-tree portfast

power inline auto

! Shut unused

interface range FastEthernet0/11 - 24

shutdown

interface vlan 10

ip address 10.10.10.5 255.255.255.0

no shutdown

ip default-gateway 10.10.10.2

line vty 0 4

transport input ssh

login local

## SW-Office-Cam — Cisco 2960 (1F Offices / NVR Zone)

Uplink to SW-1F-DIST. Carries VLANs: 10, 20, 50, 99. No RFID on this switch. Default gateway: 10.10.10.2

vlan 10

name Management

vlan 20

name Staff

vlan 50

name Security

vlan 99

name Guest

! Uplink to SW-1F-DIST

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,50,99

no shutdown

! Office PC ports — VLAN 20

interface range FastEthernet0/1 - 4

switchport mode access

switchport access vlan 20

spanning-tree portfast

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

! Camera ports — PoE, VLAN 50

interface range FastEthernet0/5 - 10

switchport mode access

switchport access vlan 50

spanning-tree portfast

power inline auto

! Shut unused

interface range FastEthernet0/11 - 24

shutdown

interface vlan 10

ip address 10.10.10.6 255.255.255.0

no shutdown

ip default-gateway 10.10.10.2

line vty 0 4

transport input ssh

login local

## SW-GF-LabCL1 — Cisco 2960 (GF Comp Eng Lab + CL1)

Direct uplink to SW-GF-DIST. Carries VLANs: 10, 30, 40, 50, 60, 99. Default gateway: 10.10.10.3

vlan 10

name Management

vlan 30

name Labs

vlan 40

name Wireless

vlan 50

name Security

vlan 60

name Access-Control

vlan 99

name Guest

! Uplink to SW-GF-DIST (direct independent link)

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,30,40,50,60,99

no shutdown

! AP port 1 (Comp Eng) — trunk: VLAN 40 + VLAN 30 + VLAN 99

interface FastEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 30,40,99

spanning-tree portfast

! AP port 2 (CL1) — trunk: VLAN 40 + VLAN 30 + VLAN 99

interface FastEthernet0/2

switchport mode trunk

switchport trunk allowed vlan 30,40,99

spanning-tree portfast

! Lab / CL1 PC ports — VLAN 30

interface range FastEthernet0/3 - 10

switchport mode access

switchport access vlan 30

spanning-tree portfast

! Camera ports — PoE, VLAN 50

interface range FastEthernet0/11 - 12

switchport mode access

switchport access vlan 50

spanning-tree portfast

power inline auto

! RFID reader port — PoE, VLAN 60

interface FastEthernet0/13

switchport mode access

switchport access vlan 60

spanning-tree portfast

power inline auto

! Shut unused

interface range FastEthernet0/14 - 24

shutdown

interface vlan 10

ip address 10.10.10.7 255.255.255.0

no shutdown

ip default-gateway 10.10.10.3

line vty 0 4

transport input ssh

login local

## SW-GF-Office — Cisco 2960 (GF Offices)

Direct uplink to SW-GF-DIST. Carries VLANs: 10, 20, 50, 99. Default gateway: 10.10.10.3

vlan 10

name Management

vlan 20

name Staff

vlan 50

name Security

vlan 99

name Guest

! Uplink to SW-GF-DIST (direct independent link)

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,50,99

no shutdown

! Office PC ports — VLAN 20

interface range FastEthernet0/1 - 4

switchport mode access

switchport access vlan 20

spanning-tree portfast

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

! Camera ports — PoE, VLAN 50

interface range FastEthernet0/5 - 10

switchport mode access

switchport access vlan 50

spanning-tree portfast

power inline auto

! Shut unused

interface range FastEthernet0/11 - 24

shutdown

interface vlan 10

ip address 10.10.10.8 255.255.255.0

no shutdown

ip default-gateway 10.10.10.3

line vty 0 4

transport input ssh

login local